## Missing NVDIA DRIVERS

While the Terraform flag install-nvidia-driver = "True" attempts to automate this, it sometimes fails on Debian if the "non-free" repositories aren't ready. We will fix this manually so your L4 GPU can finally talk to Python.This error usually means that while the software for nvidia-smi exists, the Kernel Module (the actual bridge between the OS and the L4 hardware) failed to load.

### Step 1: The "Nucleus" Fix (DKMS Rebuild)

We need to force the system to rebuild the driver specifically for your current running kernel.

# 0. See if nvida drivers are installednvidia-smi# 1. Elevate to root and enter your workspacesudo su cd /# 2. This command will overwrite your current sources.list with the exact verified repository linesecho -e "deb http://deb.debian.org/debian/ bullseye main contrib non-free\ndeb-src http://deb.debian.org/debian/ bullseye main contrib non-free\n\ndeb http://security.debian.org/debian-security bullseye-security main contrib non-free\ndeb-src http://security.debian.org/debian-security bullseye-security main contrib non-free" | sudo tee /etc/apt/sources.list && sudo apt update# 3. Run this exact command. It will purge any broken drivers, install the Tesla-grade driver (essential for the L4), and ensure the kernel headers are perfectly matched.sudo apt-get purge '^nvidia-.*' -y && sudo apt-get autoremove -y && sudo apt update && sudo apt install -y linux-headers-$(uname -r) nvidia-driver nvidia-smi nvidia-persistenced dkms && sudo modprobe nvidia# 4. Install DKMS (Dynamic Kernel Module Support)sudo apt install dkms -y# 5. Reconfigure the NVIDIA kernel modules# This forces a 're-marriage' between your kernel and the driversudo dpkg-reconfigure nvidia-kernel-dkms# 6. Grab the official NVIDIA production repo for Debian 11wget https://developer.download.nvidia.com/compute/cuda/repos/debian11/x86_64/cuda-keyring_1.1-1_all.debsudo dpkg -i cuda-keyring_1.1-1_all.debsudo apt update# 7. Install the Tesla-grade driver (Specifically for L4)sudo apt install -y cuda-drivers-535nvidia-smi

### Why this happened 🕵️

- Missing SMI: nvidia-smi is the tool that talks to the driver. If the driver isn't installed, the tool doesn't exist.

- Metadata Timing: Sometimes the Google Cloud "auto-install" script times out if the Debian mirrors are slow, leaving your machine in a half-finished state.

- Non-Free Policy: Debian requires explicit permission to use NVIDIA code.